How Otto moves ?

J. Lemaire

(Pierrefeu Mai 2021)

# Motivations

Some time ago, my granddaughter (8 years old) expressed her enthusiasm, following an initiation to robotics that she had just taken at school. This prompted me to give her an OttoDIY+ robot, to let her know a little more... and possibly wow the friends !

But, while helping to assemble it, I admit I was totally amazed by this little Robot, really brilliant by its simplicity and its possibilities, in particular by its approach using only 4 micro servos.

Having been a teacher in a computer environment in another lifetime (I'm now retired), I got caught up in the game and tried to understand how it works.

Moreover, having observed like other users that the firmware proposed to control OttoDIY+ in Bluetooth was extreme for an Arduino Nano which only has 2K of SRAM memory, I wondered if it was not possible to optimize a bit of the C ++ code provided, limiting myself here to the only problem of basic Otto movements, for simplicity.

This document exposes this little work. But before going into the details, I want to make it clear that I am not trying here to criticize the remarkable work of the designers of Otto, but just to understand its  locomotor functioning, to evoke the questions that I asked myself occasionally, and very modestly, to present the provided answers. In doing so, I am well aware of the very limited nature of this study in view of all of Otto's body language !

# Servos

Otto's movements are carried out by 4 micro-servos of the MG90S type.

They each consist of a small DC motor that, through a series of gears, can rotate an axis through an angle of approximately 0-180 °. An integrated circuit controls the motor, the angle being controlled by a potentiometer:

This integrated circuit can be controlled by a 50Hz frequency PWM signal (period T = 20ms), the value of which, also denoted PWM for simplicity, will be defined by the duration of the HIGH state over each period :

Default for a servo :

- PWMmin = 544µs => angle  0°

- PWMmax = 2400µs => angle  180°

The angle α of rotation, also called the position of the servo, will be measured as follows, the half-arm being mounted so that this angle can vary between 0 and 180 °:

To control a servo with an Arduino and on any digital pin, just use the Servo library made by Michael Margolis, which defines the Servo class:

class Servo

{

public:

Servo();

uint8_t attach(int pin);           // attach the given pin to the next free channel, sets pinMode, returns channel number or 0 if failure

uint8_t attach(int pin, int min, int max); // as above but also sets min and max values for writes.

void detach();

void write(int value);             // if value is < 200 its treated as an angle, otherwise as pulse width in microseconds

void writeMicroseconds(int value); // Write pulse width in microseconds

int read();                        // returns current pulse width as an angle between 0 and 180 degrees

int readMicroseconds();            // returns current pulse width in microseconds for this servo (was read_us() in first release)

bool attached();                   // return true if this servo is attached, otherwise false

private:

uint8_t servoIndex;               // index into the channel data for this servo

int8_t min;                       // minimum is this value times 4 added to MIN_PULSE_WIDTH

int8_t max;                       // maximum is this value times 4 added to MAX_PULSE_WIDTH

};

Here is an example of use in an Arduino sketch, for a servo connected to pin 2 :

#include <Servo.h>

Servo s;         // Servo object

void setup()

{

s.attach(2);  // Attaches the servo to pin 2

s.write(0);   // Turns 0°

}

void loop()

{

}

In the case of an Arduino Nano, 12 servos can be used in parallel and the implementation of this library takes advantage of Timer1 to generate such signals, in interrupt mode. Here are the signals observed with an oscilloscope on pin 2, when we run the previous sketch:

This interrupt control mode allows a 50Hz PWM signal to be sent to any digital output pin and to run servos in parallel, without blocking the microcontroller. But, the Servo library does not allow to control their rotation speed or even to stop them on a precise angle. Finally, note that a servo is not as precise as a stepping motor : it is therefore often necessary to trim it, to make the effective angle of the arm coincide with the angle transmitted to the write method, for example at 90° ; this possibility is also ignored in the Servo library. On the other hand, one can redefine the values ​​PWMmin and PWMmax, which is another way, fairer but not always possible, to solve the problem.

The following sketch shows that the read method returns the last angle passed to write and not the current position of the servo:

#include <Servo.h>

Servo s1,s2;

void setup()

{

s1.attach(2);

s2.attach(3);

s1.write(0);

s2.write(0);

Serial.begin(115200);

delay(1000);

Serial.println("start");

Serial.println(millis());

Serial.print("s1="); Serial.print(s1.read());

Serial.print(" s2="); Serial.println(s2.read());

s1.write(180);

s2.write(180);

}

void loop()

{

Serial.println(millis());

Serial.print("s1="); Serial.print(s1.read());

Serial.print(" s2="); Serial.println(s2.read());

delay(10);

}

Display on the serial monitor :

start

999

s1=0 s2=0

1000

s1=180 s2=180

1010

s1=180 s2=180

1021

s1=180 s2=180

1032

s1=180 s2=180

…

To end this short presentation, it should be remembered that a servo, even micro, can generate a significant current : this can exceed 200mA, when operating under constraints. In the case of Otto, it is therefore strongly recommended to supply its 4 micro-servos separately, and not only via a type 2 USB serial port, which cannot supply more than 500mA. This will also protect the Arduino Nano microcontroller and other robot components against current variations, when powered through its VIN input, even if its 5V regulator (AMS 1117 5.0) here seems quite tolerant and responsive.

To fully understand and implement an effective solution, here is Otto's default power circuit diagram:

# The solution adopted in Otto

The previous convention of measuring the angle of rotation of a servo leads to this, for all 4 Otto servos, mounted according to the assembly instructions :

- αLL => left leg

- αRL => right leg

- αLF => left foot

- αRF => right foot

Then :

- αLL>90° or αRL>90°  legs turned to the right

- αLF>90°  left foot lowered (outward)

- αRF<90°  right foot lowered (outward)

An Otto movement for a duration T ms will be specified by the data of 4 angular functions, again denoted αLL,αRL,αLF and αRF for simplicity, defined on the interval [0, T] and with values ​​in [0.180].

For example, here are the functions called by the walk (1, 1000, FORWARD) method, to move Otto forward for 1000ms, with a step to the left then a step to the right:

When 0≤t≤1000 :

- αLLt=αRLt=30sin2πt1000+90

- αLFt= 20sin2πt1000-π2+94

- αRFt= 20sin2πt1000-π2+86

These cause the robot to advance with a fluid swing where it leans successively on its left leg then on its right leg, while making them turn to the right, then to the left (this is the resistance of the ground under the flat foot which allows this) :

(L-clic to see the video)

To move it back, simply reverse the phase shifts of the feet :

To rotate it while moving forward, all you have to do is reduce the amplitude of the rotation of the leg on the opposite side to which you want to turn ; for example :

We can therefore imagine how to make it turn, moving backward this time:

Finally, if you want to make it turn on the spot, it suffices to generate a contradictory movement of the 2 legs, as for a mechanical showel :

All these angular functions are of the form :

αt=asin2πtT+φ+b

With

- a = amplitude (°)

- b = offset (°)

- φ = phase (radians)

- T = duration or period (ms)

Having noticed that several other movements could also be carried out with such functions (turn, updown, swing, tiptoeSwing, jitter, ascendingTurn, moonwalker, crusaito, flapping) , the designers of Otto used « oscillator » objects, instances of the Oscillator class, almost identical to the ServoOsc class defined in the library of the same name. It was initially developed by Juan González Gómez, as part of his Ph.D. thesis in robotics, Robótica Modular y Locomoción: Aplicación a Robots Ápodos (2008), who clearly demonstrated the importance of this concept, with an Arduino implementation, used in early versions of Otto:

class Oscillator

{

public:

Oscillator(int trim=0) {_trim=trim;};

void attach(int pin, bool rev =false);

void detach();

void SetA(unsigned int A) {_A=A;};

void SetO(unsigned int O) {_O=O;};

void SetPh(double Ph) {_phase0=Ph;};

void SetT(unsigned int T);

void SetTrim(int trim){_trim=trim;};

int getTrim() {return _trim;};

void SetPosition(int position);

void Stop() {_stop=true;};

void Play() {_stop=false;};

void Reset() {_phase=0;};

void refresh();

private:

bool next_sample();

private:

//-- Servo that is attached to the oscillator

Servo _servo;

//-- Oscillators parameters

unsigned int _A;  //-- Amplitude (degrees)

unsigned int _O;  //-- Offset (degrees)

unsigned int _T;  //-- Period (miliseconds)

double _phase0;   //-- Phase (radians)

//-- Internal variables

int _pos;         //-- Current servo pos

int _trim;        //-- Calibration offset

double _phase;    //-- Current phase

double _inc;      //-- Increment of phase

double _N;        //-- Number of samples

unsigned int _TS; //-- sampling period (ms)

long _previousMillis;

long _currentMillis;

//-- Oscillation mode. If true, the servo is stopped

bool _stop;

//-- Reverse mode

bool _rev;

};

This Oscillator class is basically associated with a servo that it can « oscillate ». It allows to define the parameters a, b, φ and T of a function α of the previous form, by calling the methods SetA, SetO, SetPh and SetT. The refresh method causes the update of the movement obtained by applying this function ; it suffices to call it periodically in the loop section and this call is not-blocking. You can also trim the servo by calling the SetTrim method, stop it at any time by calling the Stop method and reduce its consumption by calling the detach method.

From the point of view of its movements, an Otto object, here instance of the Otto9 class, is therefore essentially defined by the 4 oscillators stored in the array :

Oscillator servo[4];

All the movement functions ultimately call the oscillateServos method, passing its arrays containing the appropriate values ​​of the previous parameters ; after having taken them into account, the latter uses the refresh method of the 4 oscillators:

void Otto9::oscillateServos(int A[4], int O[4], int T, double phase_diff[4], float cycle=1) {

for (int i=0; i<4; i++) {

servo[i].SetO(O[i]);

servo[i].SetA(A[i]);

servo[i].SetT(T);

servo[i].SetPh(phase_diff[i]);

}

double ref=millis();

for (double x=ref; x<=T*cycle+ref; x=millis()) {

for (int i=0; i<4; i++) {

servo[i].refresh();

}

}

}

All this therefore very clearly implements this notion of oscillation, central in the thesis of Juan González Gómez.

Without wishing to claim to correct this implementation, it is nevertheless tempting to better distribute the different functionalities considered here :

- The notion of movement first, which could use functions other than oscillations, with possibilities of sampling, interrupting and resuming, all in a non-blocking operation mode ;

- Its application to a servo, by inheriting the functionality of the Servo class, with a possibility of trim, as in OttoDIYLib ;

- The management of several servos in parallel ;

- The implementation of this in Otto, taking into account all the movements mentioned above, at different speeds and in a fluid manner ;

- Control of Otto via a serial port and in particular the one created by its Bluetooth module.

This is what we will attempt to do in the next paragraph.

But before doing this, we would like to ask the designers of Otto a question: How were the parameters of the various oscillatory movements determined, and in particular those of the feet ?

For example, if we consider the walk method where the legs are always parallel, we can calculate the angle of inclination of one foot so that the other is flat and this can be done graphically with a drawing software (Autocad here) and a scale representation of Otto:

Thus, when the right foot is raised with an angle of 106° (when the 2 legs have an angle of 90 ° and are therefore separated as far as possible), the left foot should be lowered with an angle of 140° so that the right foot either flat. However, the latter is set at 114° in the walk oscillation. Why ?

# Classes Movement, ServoE and TServoE<N>

## Movement function

To vary the angle of a servo over time, we have seen that it suffices to define a function α over an interval [0, T] ; if the movement starts at timet0, the angle will be equal to αt at time t0+t.

This function α will generally be characterized by parameters ; this is the case, for example, for an oscillation, specified by the parameters a, b, φ and T :

αt=asin2πtT+φ+b  with 0≤t≤T

In C ++ , if the times are evaluated in milliseconds, we can therefore naturally consider the type <double (double)> for a movement function.

If we have a version at least equal to 11 of this language, there is a very direct way to implement such a function, with parameters : it suffices to consider a Lambda function whose parameters of the capture list (between []) correspond to the aforementioned parameters, cast with std::function to ultimately obtain a function of the type sought ; for example, for an oscillation :

std::function<double(double)> [a,b,phi,T](double t){return a*sin(2*PI*t/T+phi)+b;};

Unfortunately this powerful mechanism does not seem possible with an Arduino Uno or Nano, for which the Arduino IDE, which uses the avr-G ++ 7.3.0 compiler, does not know about std::function. This restriction is explained by the very low SRAM memory available on their ATMega328P processor : 2KB only !

We therefore implemented another technique, less compact but much more economical in processor resources, based on a class of parametric functions and a pointer to one of them. These functions, although classic, look like Lambda functions.

Class Movement (files Movement.h and Movement.cpp) :

#define TS 20            // Sampling period(ms) to update movements

enum StateMovement {active, stopped, inactive};

class Movement

{

typedef double (Movement::*PtrMovementF)(double);

// Type of pointer on a movement function

public:

// Constructor

Movement();

// Selection methods (NB : speed (unit/s) >0)

void selAff(double a, double b, double T, unsigned nRep = 0);

void selOsc(double a, double b, double phi, double  T, unsigned nRep = 0);

// Access to private fields

double T() const {return _T;}

void setT(double T);

StateMovement stateMov() const {return _stateMov;}

void setStateMov(StateMovement stateMov) {_stateMov = stateMov;}

double operator()(double t) {return ((*this).*_pMovF)(t);} // Selected function

// Sampling with TS period

void start();

bool update();

void stop();

double val() const {return _val;}

private:

// Movement functions

double Aff(double t) {return _a*(1-t/_T)+_b*t/_T;}       // Affine

double Osc(double t) {return _a*sin(2*PI*t/_T+_phi)+_b;} // Oscillation

// Parameters

double _a;

double _b;

double _phi; // (radians)

double _T;   // Period(ms) > 0 (no control)

// Other fields

PtrMovementF _pMovF;        // Pointer on the selected function

StateMovement _stateMov;    // State movement

unsigned int _nRep;         // Number of repetitions

unsigned int _iUse;         // Movement use number (0, 1, ..., _nUse)

unsigned long _startTime, _endTime, _previousTime, _stopTime;

double _val;                // Sampled value

};

It only implements 2 movement functions here, provided as an example, both of which will be used later:

- αt=a(1-tT)+btT           (affine)

- αt=asin2πtT+φ+b  (oscillation)

But it's very easy to add more!

To select one of them, simply call the corresponding Sel... method, passing it its parameters, as well as a possible repetition factor.

Therefore, by using the operator(), we can obtain the values ​​of this function for 0≤t≤T, or beyond if it is repeated.

This class also makes it possible to sample a movement with a period TS, fixed here at 20ms : for that, it suffices to call initially the start method, then update in the loop ; the latter is non- blocking and returns true when a new sample is available. In both cases, the val method provides the value of the movement.

You can also interrupt the movement by calling stop, then resume it by calling start again. Note that the update method returns true during the interrupt and that val does not change.

Sketch TestMovement.ino :

// TestMovement.ino

#include <Movement.h>

Movement m;

unsigned long oldTime, newTime;

byte state;

void print(double x)

{

Serial.print(x); Serial.print('\r');

}

void setup()

{

Serial.begin(9600);

delay(100);

m.selOsc(2, 4, 0, 1000, 2);  // Osc[2,4,0,1000] 3 times

m.start();

print(m.val());

oldTime = millis();

state = 0;

}

void loop()

{

if (m.update()) print(m.val());

newTime = millis();

if (newTime-oldTime >= 500)

{

oldTime = newTime;

if (state==0) {m.setT(500); state++;}       // Period/2

else if (state==1) {m.stop(); state++;}     // Stop

else if (state==2) {m.start(); state++;}    // Restart

}

}

We consider here an oscillation, Osc[2, 4, 0, 1000], 3 times repeated, and sampled at 20ms. We start by dividing by 2 the period of 1000ms after 500ms, then we stop the movement after 500ms, and we finally resume it after 500ms

Display of the results with Serial Oscilloscope :

(NB : horizontal unit = 40ms)

We can therefore very easily select, sample, interrupt and resume a movement, without having to manage the sampling.

## Extension of the Servo class

We are going to extend, by inheritance, the possibilities of the Servo class by offering it to be able to control the movement of a servo over time, with one of the parametric functions previously defined, and this in the least blocking way possible.

However, proceeding in this way has a small drawback : that of not knowing the exact position (= angle) of the servo, if the movement is interrupted, and this can be useful in certain movement functions. We will therefore permanently store this position in the _pos field and implement a move method similar to Servo::write, to update it. This will also allow us to integrate the trim correction :

void move(double pos);

Initially, we wanted to introduce a systematic waiting period at the end of this method, to ensure that the target position is achieved. But we gave it up, because it disrupted the sampling. It will therefore be advisable to assess the risk of this situation occurring, and if necessary call on delay immediately afterwards, when it is called directly.

For this reason, it will be better to use instead :

void move(double pos, double T);

which will control the movement with a duration equal to T ms, using Aff[_pos, pos, T] as the movement function. You will have to choose T large enough so that for each sampling (all TS = 20ms), we have plenty of time to complete the rotation of the arm, knowing that in 20ms we can rotate a maximum of 12°.

Obviously, as this class will be able to use the Aff and Osc parametric functions of the Movement class to control the movements of the servo over time. This can be done in a non-blocking way with the following methods which overload the methods of the same name of the Movement class and can be used identically :

void start();

bool update();

void stop();

Class ServoE (files ServoE.h and ServoE.cpp) :

class ServoE : public Servo, public Movement

{

public:

// Constructors

ServoE();

ServoE(unsigned pin, double trim = 0.);

// Access to private fields (only necessary)

void setPin(unsigned pin) {_pin = pin;};

void setTrim(double trim) {_trim = trim;};

void setPos(double pos) {_pos = pos;};

double pos() const {return _pos;};

// Methods

void attach();

void detach();

// Blocking movement

void move(double pos);           // Uncontrolled

void move(double pos, double T); // Controlled

// Non-blocking controlled movement

void start();

bool update();

void stop();

bool isInactive();

private:

unsigned _pin;         // Arduino pin where the servo is connected

double _trim;          // Trim value on writing (°)

double _pos;           // Current position (°)

};

Sketch TestServoE.ino :

// TestServoE.ino

#include <ServoE.h>

ServoE s(3,-5);  // Servo on pin 3 with trim -5

unsigned long oldTime, newTime;

unsigned sta;

void print(double x)

{

Serial.print(x); Serial.print('\r');

}

void setup()

{

Serial.begin(9600);

delay(100);

s.attach();

// Oscillation (x4) with 3000ms period

s.selOsc(90, 90, 0, 3000, 3);

s.start();

print(s.pos());

oldTime = millis();

sta = 0;

}

void loop()

{

if (s.update()) print(s.pos());

newTime = millis();

if (newTime-oldTime >= 2000)

{

oldTime = newTime;

if (sta == 0) {s.setT(1000); sta++;}    // Period/3 at 2s

else if (sta == 1) {s.stop(); sta++;}   // Stop at 4s

else if (sta == 2) {s.start(); sta++;}  // Restart at 6s

}

if (s.isInactive()) s.detach();

}

We consider a servo on pin 3, trimmed at -5°. Its movement is an oscillation defined by Osc [90, 90, 0, 3000], repeated 3 times and sampled at 20ms. We start by dividing its period at 2s by 3, then we stop the movement at 4s, then we resume it at 6s.

Display of the results with Serial Oscilloscope :

(NB : horizontal unit = 100ms)

## Movement of several servos in parallel

The TServoE<N> class responds to this problem by defining classical methods on an array of N servos, all of type ServoE.

Class TServoE<N> (file TServoE.hpp)

/***********************************************************************************

Class TServoE<N>

************************************************************************************

Table of N extended servos (template)

Public methods :

TServoE() : constructor

TServoE(unsigned tPin[], double tTrim[] = NULL) : constructor

ServoE &operator[](unsigned i) : access to servos

void attach() : attach

void detach() : detach

void move(double tPos[], double T) : blocking controlled parallel movements

void start() :  start or restart non-blocking controlled parallel movements

bool update() : update non-blocking controlled parallel movements

void stop() : stop non-blocking controlled parallel movements

bool isInactive() : test if all the servos are inactive

*/

Since this is a template class, it is declared and implemented in a single file, TServoE.hpp, of which we only provide comments here.

The operator[] gives a reference on each servo in the table. By using it, you can for example select a movement function for it.

All the movement methods make the N servos turn in parallel by controlling them and there again, we have start, update and stop methods, to do it in a non-blocking way.

Sketch TestTServoE.ino :

// TestTServoE.ino

#include <TServoE.hpp>

unsigned tPin[] = {2,3};

double tTrim[] = {-12,-5};

TServoE<2> tS(tPin, tTrim);

unsigned long oldTime, newTime;

unsigned sta;

void print(double x, double y)

{

Serial.print(x); Serial.print(','); Serial.print(y); Serial.print('\r');

}

void setup()

{

Serial.begin(9600);

delay(100);

tS.attach();

// Blocking controlled parallel movements to start positions

double tPos[] = {45, 180};

tS.move(tPos, 500);

// Non-blocking controlled parallel movements

tS[0].selOsc(45, 45, 0, 2000, 3);

tS[1].selOsc(45, 135, PI/2, 3000, 3);

tS.start();

print(tS[0].pos(), tS[1].pos());

oldTime = millis();

sta = 0;

}

void loop()

{

if (tS.update()) print(tS[0].pos(), tS[1].pos());

newTime = millis();

if (newTime-oldTime >= 2000)

{

oldTime = newTime;

if (sta == 0) {tS[1].setT(1000); sta++;}   // Period/3 at 2s

else if (sta == 1) {tS[1].stop(); sta++;}  // Stop at 4s

else if (sta == 2) {tS[1].start(); sta++;} // Restart at 6s

}

if (tS.isInactive()) tS.detach();

}

Display of the results with Serial Oscilloscope :

(NB : horizontal unit = 200ms)

Connection diagram with an Arduino Uno :

# Implementation with Otto

With these 3 classes Movement, ServoE and TServoE<N>, it is then very easy to define a Robot class to model Otto's movements : you just have to make it inherit from TServoE<4> !

Class Robot (files Robot.h and Robot.cpp) :

enum Part {LL, RL, LF, RF};

enum Direction {FORWARD, BACKWARD};

enum Side {LEFT, RIGHT};

enum Speed {SLOW, NORMAL, RAPID};

/***********************************************************************************

Class Robot

************************************************************************************

Robot class for OttoDIY with only legs and feet movement functionnalities

*/

class Robot : public TServoE<4>

{

public:

Robot(unsigned pinLL, unsigned pinRL, unsigned pinLF, unsigned pinRF, bool calibrated = true); // Constructor

void setSpeed(Speed spd);                        // To change the movement speed

void move(double tPos[]);                        // Blocking controlled parallel movements to tPos

void home();                                     // Blocking controlled parallel movements to {90,90,90,90}

void walk(Direction dir, unsigned int nRep = 0); // To start a walk non-blocking movement

void turn(Side sid, unsigned int nRep = 0);      // To start a turn non-blocking movement

void walk_turn(Direction dir, Side sid, unsigned int nRep = 0); // To start a walk_turn non-blocking movement

private:

Speed _spd;

void setMov(double tA[], double tB[], double tPhi[], unsigned int nRep);

};

Otto having 4 servos, we defined a constructor to which it is necessary to indicate the numbers of the used pins : 2, 3, 4, 5 by default. As in OttoDIYLib, its last argument, calibrated, indicates whether we can read the trim values ​​from EEPROM.

3 movement methods are provided as an example :

- The walk and walk_turn methods are almost identical to the walk and turn methods of the Otto9 class, with the addition of the possibility of turning backwards for the second ;

- The turn method corresponds here to a rotation of the robot in place, to the left or to the right.

Their implementations are analogous to those of the Otto9 class ; for example :

void Robot::turn(Side sid, unsigned int nRep = 0)

{

double phi;

switch (sid)

{

case LEFT: phi = -PI; break;

case RIGHT: phi = 0; break;

}

float tA[4] = {15, 15, 20, 20};

float tB[4] = {105, 75, 94, 86};

float tPhi[4] = {-PI/2, -3*PI/2, phi, phi};

setMov(tA, tB, tPhi, nRep);

}

with :

void Robot::setMov(double tA[], double tB[], double tPhi[], unsigned int nRep)

{

move(tB);

double x = 0.1; // Need T > 0 !

for (unsigned i=0; i<4; i++) x = max(x, abs(tA[i]));

double T = 4000*x/tSpeed[_spd];

for (unsigned i=0; i<4; i++) (*this)[i].selOsc(tA[i], tB[i], tPhi[i], T, nRep);

start();

}

It should be noted that such movements therefore break down into :

- An affine blocking movement to rotate the servos to the starting positions of the oscillatory movement, in order to increase the fluidity of the movement ;

- A non-blocking oscillatory movement, with the appropriate parameters.

These 2 movements are controlled by a speed parameter _spd, from which we deduce the periods T, common for the 4 servos of the robot, which are therefore well synchronized. The setSpeed ​​method allows you to change it « hot ».

Sketch TestRobot.ino :

// TestRobot.ino

#include <Robot.h>

Robot r(2, 3, 4, 5);

void setup()

{

r.attach();

r.home();

r.walk(FORWARD, 5);

}

void loop()

{

r.update();

if (r.isInactive())

{

r.home();

r.detach();

}

}

# Controlling the robot via a Bluetooth connection

## Classes SerialCommand and RobotSC

The essential thing to understand here is that the problem is to control Otto via a TTL serial link, for example the one established with an USB connection, using the Serial object, instance of the HardwareSerial class.

Indeed, Otto's Bluetooth module is connected to pins 12 (RX) and 11 (TX) of the Arduino Nano, with establishment of a TTL serial link on them, using an instance of the SoftwareSerial class.

However, if we look at the definitions of these 2 classes :

class HardwareSerial : public Stream

class SoftwareSerial : public Stream

we see that they both inherit from the Stream class and notably implement its virtual methods :

virtual int available() = 0;

virtual int read() = 0;

Thanks to the polymorphism of C ++, we can treat the two cases in a generic way, by considering an instance (in fact a reference to avoid an unnecessary copy) of the Stream class, which can therefore accept an instance object of the HardwareSerial class as well as Serial, as an instance object of the SoftwareSerial class (see below), and of course call the previous methods. This is what we will do in the SerialCommand class defined as follows.

Class SerialCommand (files SerialCommand.h and SerialCommand.cpp) :

class SerialCommand

{

public:

SerialCommand(Stream& serial, int nCmd, Command tCmd[]);

char typ() const {return _typ;}

unsigned opt() const  {return _opt;}

void resetParse();

bool parseCommand();

private:

Stream& _serial;              // Serial port used for commands

unsigned _nCmd;               // Number of acceptable commands

Command *_tCmd;               // Array of acceptable commands (as reference)

bool _okCommand;              // True if a command as been correctly parsed

char _typ;                    // Type of this command

unsigned _opt;                // Option of this command

unsigned _nParse;             // Number of parsed characters

};

The purpose of this class is to analyze the commands received on the serial Stream passed to its constructor. These must respect the following syntax and semantics:

Syntax :

<command> = <typ><opt>#

<typ> = <letter>

<letter> = A|B|...|Z

<opt> = <digit>|<digit><digit>

<digit> = 0|...|9

Semantic :

(<typ>,<opt>) in tCmd

The tCmd array of acceptable commands, a parameter of the constructor, is here defined by :

Command tCmd[] =    // Acceptable commands

{

{'C',2,{1,0}},    // Contact : 1=>on, 0=>off

{'M',9,{0,1,2,3,4,13,14,23,24}},

// Movement : 0=>home, 1/2=>walk FORWARD/BACKWARD, 3/4=>turn LEFT/RIGHT, 13=>walk FORWARD and turn LEFT, etc.

{'I',2,{1,0}},    // Interruption : 1=>on, 0=>off

{'S',3,{1,2,3}}   // Speed : 1=>SLOW, 2=>NORMAL, 3=>RAPID

};

each being of type Command :

struct Command {char typ; unsigned nOpt; unsigned tOpt[MAX_OPTIONS];};  // Acceptable command

For instance :

- M0# => home

- M1# => walk forward

- M3# => left turn (at the place)

- M13# => walk forward and turn left

- Etc.

As we do not always control the production of ‘\ n’ (line feed) or ‘\ r’ (carriage return), these characters will be ignored and we preferred to use ‘#’ to indicate the end of a command.

The main method of this class is responsible for decoding the commands received :

bool parseCommand();

Its implementation is that of a classic interpreter, which returns true on receiving one of them ; all you have to do is call the typ and opt methods to find out their type and option.

And that's what we do in the unique processCommand method of the RobotSC class, which inherits from the Robot class and the previous class.

Classe RobotSC (files RobotSC.h and RobotSC.cpp) :

class RobotSC : public Robot, SerialCommand

{

public:

RobotSC(int pinLL, int pinRL, int pinLF, int pinRF, Stream& serial, bool calibrated = true);

void processCommand();

};

void RobotSC::processCommand()

{

if (parseCommand())

{

resetParse();

unsigned _opt = opt();

switch (typ())

{

case 'C':

if (_opt==1) attach();

else if (_opt==0) detach();

break;

case 'M':

if (_opt==0) home();

else if (_opt==1) walk(FORWARD, REP);

else if (_opt==2) walk(BACKWARD, REP);

else if (_opt==3) turn(LEFT, REP);

else if (_opt==4) turn(RIGHT, REP);

else if (_opt==13) walk_turn(FORWARD, LEFT, REP);

else if (_opt==14) walk_turn(FORWARD, RIGHT, REP);

else if (_opt==23) walk_turn(BACKWARD, LEFT, REP);

else if (_opt==24) walk_turn(BACKWARD, RIGHT, REP);

break;

case 'I':

if (_opt==1) stop();

else if (_opt==0) start();

break;

case 'S':

if (_opt==1) setSpeed(LOW);

else if (_opt==2) setSpeed(NORMAL);

else if (_opt==3) setSpeed(RAPID);

break;

}

}

}

Note here the resetParse call to reset the interpreter, as well as the constant value of the REP parameter, set to 100 to simplify the sending of commands.

## Controlling the Robot with a serial monitor, via an USB link

Sketch TestRoboSC.ino :

// TestRobotSC.ino

#include <RobotSC.h>

RobotSC r(2, 3, 4, 5, Serial);

void setup()

{

Serial.begin(9600);

delay(100);

}

void loop()

{

r.processCommand();

r.update();

}

Here is an example of commands transmitted with the Coolterm  serial monitor, to rotate the robot around a cube :

## Controlling the Robot with a Smartphone, via a Bluetooth link

For the Arduino sketch, as announced, you just need to replace Serial with an instance, BTSerial here, of the SoftwareSerial class.

Sketch TestRobotBT.ino :

// TestRobotBT.ino

#include <RobotSC.h>

#include <SoftwareSerial.h>

SoftwareSerial BTSerial(11,12);

RobotSC r(2, 3, 4, 5, BTSerial);

void setup()

{

BTSerial.begin (9600);

delay(100);

}

void loop()

{

r.processCommand();

r.update();

}

A Bluetooth connection must then be established between the module provided here and the Smartphone.

This « Low Energy 4.0 » Bluetooth module is compatible with an MLT-BT05 and built around a CC2541 component from Texas Instrument (or clone ...). It can be configured with AT commands .

The easiest way to set it up is to use the Arduino IDE serial monitor, connected at 9600 baud, and the Otto_BLETest.ino sketch, provided in OttoDIYLib.

Here are the responses obtained by sending the following AT commands (with the factory settings, only the serial port speed needs to be changed):

- AT+DEFAULT             // Restore factory settings

- AT+NAME                  // Display name

- AT+VERSION             // Display version

- AT+ROLE                    // Display role (NB : 0 => Slave)

- AT+TYPE                    //  Display type (NB : 0 => no password)

- AT+BAUD4                //  Sets serial port speed (NB : 4 => 9600 bauds)

The module can then be recognized on a Smartphone accepting the Bluetooth Low Energy 4.0 protocol.

To be convinced, after activating this protocol, all you have to do is install the Android Serial Bluetooth Terminal application, available on Google Play:

If we now install the TestRobotBT.ino sketch in the robot, we can control it as we did previously with a serial monitor.

To make it easier to send commands, you can assign macros and labels to application buttons.

If you want a more « fun » interface, you can use the Bluetooth Electronics application, and build the interface by dragging and dropping its components :

Here again, the Bluetooth module is recognized when you activate Bluetooth in the Smartphone:

And then all you have to do is to compose a screen in « Edit » mode where, again, you can assign macros to the elements of the interface :

then switch to « Run » mode to control the robot.

Note, however, a few bugs, especially if we use ‘\ n’ as end of line : this is why we preferred ‘#’.

Remarks

- The executables produced with the 2 previous sketches are largely compatible with the memory possibilities of an Arduino Nano: they only occupy about 30% of the flash memory and the available SRAM memory ;

- There is therefore room to take into account other sensors : microphone, capacitive sensor (sensitive to touch), distance sensor (sonar), etc. They can be integrated as we just did with a serial port, since the latter is used here as a character « sensor ».

- This functionality can also be extended to the case of a text file or a network flow of commands, since this would still amount to implementing class instance objects inheriting from the Stream class. But here, the Blockly approach proposed with Otto is much more attractive and it would not be very difficult to adapt it to produce code compatible with our Robot class.

- The simplicity of this implementation is only possible with non-blocking processes, both at the level of the servos and at the level of the sensors. Otherwise, we would have to resort to interrupt processing, which we wanted to avoid here so as not to disturb the proper functioning of the timers and in particular timer1, used in the Servos library.

# Conclusion

This study allowed us to understand how Otto moves and in particular the importance of the oscillatory movements studied by Juan González Gómez.

To meet the objectives stated at the end of paragraph 3, we have defined an Arduino library of C ++ classes (in green below), with the following dependencies, our primary concern always being to implement the targeted functionalities as soon as possible, to so as to simplify the dependent classes as much and in the most general way possible.

In this simplified diagram, we can see the importance of the Movement class on which all other classes depend. It generically defines the notion of movement, without specifying the specific function chosen, and it encapsulates the entire process of sampling a movement, a key notion to then achieve controlled, non-blocking and parallel movements on several servos.

We have used all the power of C ++ and object-oriented programming (inheritance, composition, polymorphism, genericity, pointer to a class method, operators, template) to simply implement the targeted functionalities. This allowed us to produce very light executables, well suited to the low memory capacities of the Arduino Nano contained in Otto.

Tools such as WinPython and Serial oscilloscope have been used to produce graphics : they deserve to be known !

As our goals were only for Otto's leg and foot movements, we didn't want to take into account all of Otto's features and especially those related to its sensors. But, with the concepts implemented, this would not be difficult.

Finally, we would like to thank all of Otto's designers for inspiring us with this work and in particular Camilo Parra Palacio for the information he kindly transmitted to us.

Named only Otto, to simplify.

Département Informatique - Institut Universitaire de Technologie de Nice-Côte d’Azur, especially.

Micro Servo Motor MG90S - Tower Pro (electronicoscaldas.com)

Servos Explained - SparkFun Electronics

Pulse-width modulation - Wikipedia

GitHub - arduino-libraries/Servo: Servo Library for Arduino

An Arduino Nano, with an ATMega328P, has 3 programmable timers, timer0 (8bits), timer1 (16bits) and timer2 (8bits), which by default control the PWM outputs on pins 5/6 (with timer0 to 980Hz), 9/10 (with timer1 at 490Hz) and 3/11 (with timer2 at 490Hz), these can be generated with the analogWrite (…) method.

With time zoom.

Approximatively 300ms for 180° when 5V powered.

detach() stops the rotation, without maintaining an angle.

Graphic made with WinPython (WinPython download | SourceForge.net) and the program :

# Plot.py

# Libraries

from numpy import pi, sin, linspace

import matplotlib.pyplot as plt

# Time values

t = linspace(0, 1000, 100)

#Plots

plt.plot(t, 30*sin(2*pi*t/1000)+90, label="αLL(t)=αRL(t)=30sin⁡(2πt/1000)+90")

plt.plot(t, 20*sin(2*pi*t/1000-pi/2)+94, label="αLF(t)=20sin⁡(2πt/1000-π/2)+94")

plt.plot(t, 20*sin(2*pi*t/1000-pi/2)+86, label="αRF(t)=20sin⁡(2πt/1000-π/2)+86")

plt.axhline(y=90, linewidth=1, color='k')

plt.xlabel("Time (ms)")

plt.ylabel("Angles")

plt.title("Otto walk FORWARD movement")

plt.legend()

plt.show()

Cf. Otto9.h and Otto9.cpp in GitHub - OttoDIY/OttoDIYLib: Latest Libraries for Otto DIY robots

Cf. Oscillator.h and Oscillator.cpp in GitHub - OttoDIY/OttoDIYLib: Latest Libraries for Otto DIY robots

GitHub - fitzterra/ServoOsc: Servo oscillator library for Arduino

Juan Gonzalez:Tesis - WikiRobotics (iearobotics.com)

GitHub - Obijuan/ArduSnake: Arduino modular snake robots library. Generate the locomovement of snake robots easily!

GitHub - bqlabs/zowi: An open-source and fully hackable biped robot.

We will pass it the value of the angle (read in the EEPROM) resulting from the robot calibration procedure.

Cf. Otto9.h in GitHub - OttoDIY/OttoDIYLib: Latest Libraries for Otto DIY robots

Via the _execute method.

Cf. Otto9.cpp in GitHub - OttoDIY/OttoDIYLib: Latest Libraries for Otto DIY robots

Arduino STL libraries, such as ArduinoSTL and StandardCplusplus, also do not support the std::function. Note however the existence of an STL library which implements std::function (GitHub - fopeczek/arduino-function_objects), but for platformIO. Preferring to stay within the framework of the Arduino IDE, we have not tested it.

NB : 0 repetition => the movement is used 1 time.

https://x-io.co.uk/serial-oscilloscope/ (NB : the graphic color has been changed to make it more readable).

Here logic level = 5V for the Arduino Nano, the line being idle in the LOW state (0V).

An Arduino Uno or Nano has only one hardware serial port, connected to pins 0 (RXD) and 1 (TXD) and used during a USB connection. It is for this reason that these pins are not connected to a component of the Robot. However, the SoftwareSerial library (Arduino - SoftwareSerial) allows this functionality to be extended by software by creating other serial ports on other pins, but with restrictions.

In the Arduino IDE installation folder:

- <Arduino>/hardware/arduino/avr/cores/arduino/HardwareSerial.h

- <Arduino>/hardware/arduino/avr/libraries/SoftwareSerial/src/HardwareSerial.h

Here MAX_OPTIONS = 9.

http://freeware.the-meiers.org/ with Line Mode and Local Echo as options.

For instance : Bluetooth Module with Arduino (AT-09, MLT-BT05, HM-10) — Maker Portal (makersportal.com)

MLT-BT05-AT-commands-TRANSLATED.pdf (wlu.ca)

Xiaomi MI9 SE with Android 10 (MIUI 12.0.4) here.

Serial Bluetooth Terminal – Applications on Google Play

Bluetooth Electronics (keuwl.com)

With Localisation…

Not UML ! Blue arrows => inheritance and red arrow  => composition.

But limited to those taken into account by the Arduino IDE compiler for an Arduino Nano.